2024 Badge Final Assembly

1.44” TFT LCD Installation

- Solder 2.54mm header into bottom side of TFT LCD

- Install standoffs into P2 on the top side of the PCB

- Insert TFT LCD pins into P2 Receptacle

- Attach TFT LCD to standoffs using screws

Micro Lipo Module Installation

- Solder 2.54mm header into bottom side of Micro Lipo Module

- Install standoffs into P5 on the top side of the PCB

- Insert Micro Lipo module pins into P5 receptacle

- Attach Micro Lipo module to standoffs using screws

Electronic Paper Display (EPD) Installation:

- Apply the double sided adhesive foam pad to P3 outline on the top side of the PCB

- Install EPD into P3 connector on the bottom side of the PCB

- Wrap EPD flat printed circuit cable around the PCB

- Gently press EPD into the double sided adhesive foam pad on top side of the PCB

Battery Installation:

- Set U1 switch to the 5V/Off position

- Route the battery connector through the hole in the PCB above P5

- Attach battery to micro lipo module

- Turn on assembly for testing by setting U1 switch to the BATT position

Backing Plate Installation:

- Apply double sided adhesive foam pad to the battery

- Remove protective film from double sided adhesive foam pad

- Mount battery on PCB bottom side over the Offensive Summit graphic

- Install Standoffs onto backing plate

- Mount PCB assembly to the backing plate